Hello everyone! I’ll be presenting my research group’s work on “New complementarity formulations for nonsmooth process models,” specifically for the purpose of root-finding and optimization of piecewise-affine functions in abs-normal form.

Our main motivation here is to handle non-smoothness. Traditional numerical methods perform poorly on non-smooth functions.

- For one, descent directions become difficult to find, especially when the negative of a subgradient of said nonsmooth function may not necessarily be a descent direction – as is the case with the example function shown. The set of subgradients near the minimum result in increasing descent directions.

- Other issues are caused by a lack of differentiability and the presence of multiple local minima.

Nonsmooth functions commonly arise in:

- Modeling discrete-continuous phenomena

- Modelling safety mechanisms like relief valves

- Embedded optimization problems

- and Neural networks with ReLU activations

Now, our root-finding approach depends on something called an absolute normal form. This is essentially a way to represent continuous piecewise-affine functions as a composite of affine functions and absolute-value operations in some vector-matrix form as shown on the screen. It’s similar to a forward-automatic differentiation procedure.

This is best illustrated with the following example. Let’s consider this function f as the affine mapping of some piece-wise affine function. To construct an abs-normal form for f, we must:

- First build a vector z of the absolute-value arguments in f ordered sequentially to mirror the forward-automatic differentiation procedure. Like a recipe!

- From there, f can be written in terms of x and z to produce the matrix at the bottom of the screen.

- [Next Slide]: If you look carefully, this matrix can be partitioned into the coefficients from the given abs-normal form definition.

Established root-finding approaches per Griewank et al., solve for the function f using a linear complementary problem.

Assuming the (i) same number of inputs as outputs, and that (ii) J is nonsingular, you can define some matrices S and c. Then the root is solved using the indicated mixed linear complementarity problem where u and w are components of the max functions.

The problem is simplified to a linear complementarity problem if I-S is nonsingular.

Here complementarity-based solvers like PATH are useful.

Our group has developed new root-finding approach that takes advantage of the triangularity to reformulate the complementarity problem with less restrictive assumptions.

Given the following newly defined matrices and vectors, the root of function f can be solved using the new indicated mixed linear complementarity problem. Note we no longer require a nonsingular J or for the number of inputs to be the same as the number of outputs. There are also fewer overall variables and equations in this set-up. This is compatible with the PATH solver.

However, if we can assume those restrictive assumptions again under this new problem setting, then the problem actually simplifies to a linear complementarity problem!

Just as a sidenote, from our experience, J generally seems to be nonsingular when all the absolute variable terms z contribute in some meaningful way to f. That is, to produce non-zero answers.

Ok so that’s root-finding. Now, to minimize f instead, we can construct this equivalent linear program with complementarity constraints. This can be solved using a Branch-and-Cut approach.

Further, if we know the bounds on the optimal x and w, then we can use big-M constraints to rewrite this optimization program into a MILP.

Alrighty. Now to confirm the existence of an unconstrained global minima for any piecewise affine function, we need to examine horizon functions.

A horizon function is the mapping of some continuous function f that simply put describes how f behaves when the norm of x is large. It’s like zooming out infinitely far so that the function directional features are simplified.

So, for this example function, the solid line shows the original function where-as the horizon mapping shown by the dotted line flattens out its bumps and divots.

Ok, now since any horizon function is positively homogenous, it always has a point at the origin. Thus, the greatest lower bound (infimum) of the horizon function must be either 0 or negative infinity. Therefore, a global minimum exists if the greatest lower bound is 0 for any function f!

Now, for piece-wise affine functions, the horizon function is also piece-wise affine and can be represented in abs-normal form. So, we can set up an equivalent mixed-linear complementarity problem that tries to locate a solution for the horizon function plus 1 = 0. So long as a solution cannot be located, the existence of a global minimum is verified.

Our group has implemented these new complementarity formulations in Julia using PATH Solver and BARON.

- We can see how the root of this piecewise function can be located successively using both MLCP and LCP formulations.

- And how the both the existence of a global minimum and the minimum itself for this alternate function can also be located.

Ok now lets quickly go over the computational cost for root-solving problem of randomly generated piecewise affine functions.

- If J is a zero matrix, then PATH may fail to solve the complementarity problem at larger n whereas BARON does not. However, PATH outperforms BARON at all n.

- If J is nonsingular, then PATH may fail to solve when n is greater than 100. Do note that at this point, PATH can calculate the solution in less than the fraction of a second compared to BARON.

- The same can be said for the minimization of f for some variations of the piecewise function.

Ok! Our final section consists of root-finding formulations for ReLU neural networks. Here, we define a function g as the linear relationship between some variables x and y – where y is represented by a neural network.

Our goal here is to solve two main problem cases.

- The first involving solving for the root of a neural network. Where our motivation is locating the steady state of some system.

- The second will involve something called a neural ODE.

Alrighty, now the neural network architecture itself is a function composition of affine and activation functions. So, consider a network of L layers.

- We start with some initial nodes at layer 1.

- Each node on layer 2 is described as a linear combination of the nodes on the previous layers.

- Then each node is put through an activation function max0,x to add nonlinearity to the system.

Since the max function can be written using absolute notation, the neural network can be rewritten as a piecewise affine composite function. Therefore, we can represent in the indicated modified abs-normal form to solve for the root of the function g.

To calculate the root of the neural net, we can set P to 0 and solve accordingly. Our current implementation shows we can locate a local solution of the root.

Now, our second problem case, neural ODEs. Neural ODEs are just an initial value ODE system whose derivatives are represented using neural networks. Neural networks are used when the problem dynamics are too complicated to represent directly mathematically.

- To solve this, we set P,Q,r accordingly to describe the Euler method for a neural ODE. Solving g would then just be minimizing a root finding equation at N timesteps.

- Basically, consider some initial x value at time t0. The neural network is used to calculate the derivative at the next t1. This is then used to calculate x at that time t1. This process is repeated at each timestep.

Now, our current implementation, given some set of neural network parameters representing some ODE, can automatically set up the abs-normal coefficients and solve the system at each time-step within some range using the complementarity solver.

We can see how for simple nonlinear ODEs, the estimation of the ODE trajectory maps accurately to the original raw data used to construct the neural ODE.

In conclusion:

- Piece-wise affine approximations for nonsmooth functions are useful. They can be constructed using a forward-automatic differentiation procedure and represented in abs-normal form.

- We have presented new root finding and optimization approaches for these functions, with fewer variables and equations than the established approach.

- We have also presentation formulations for optimization problems with ReLU neural networks.