Hello everyone! I’ll be presenting my research group’s work on “New complementarity formulations for nonsmooth process models,” specifically for the purpose of root-finding and optimization of piecewise-affine functions in abs-normal form.

Our main motivation here is to handle non-smoothness. Traditional numerical methods perform poorly on non-smooth functions.

- For one, descent directions become difficult to find. Consider this nonsmooth function here where at the bottom of this ‘valley’, most directions are ascent directions. Whereas, in a smooth function, at-least half would be descent directions.

- Reliable methods, like steepest descent with exact line search, are also more likely to fail. For example, with this function here, if you start at this point, the method will converge to this point rather than any meaningful local minimum.

Nonsmooth functions commonly arise in:

- Modeling discrete-continuous phenomena

- Modelling safety mechanisms like relief valves

- Embedded optimization problems

- and Neural networks with ReLU activations

Now, our root-finding approach depends on something called an absolute normal form. This is essentially a way to represent continuous piecewise-affine functions as a composite of affine functions and absolute-value operations in some vector-matrix form as shown on the screen. It’s similar to a forward-automatic differentiation procedure.

This is best illustrated with the following example. Let’s consider this function f as the affine mapping of some piece-wise affine function. To construct an abs-normal form for f, we must:

- First build a vector z of the absolute-value arguments in f in the order that you would evaluate them by hand – similar to a recipe!

- From there, f can be written in terms of x and z to produce the matrix at the bottom of the screen.

- [Next Slide]: If you look carefully, this matrix can be partitioned into the coefficients from the given abs-normal form definition.

- Keep in mind that L here is lower triangular. Since inner functions are evaluated first, any later z variables depend on earlier z’s but never the other way around. So, we end up with a lower triangular L. This will be really important later.

Absolute-normal form problems are solved using a complementary approach. Let’s briefly go over the notation for complementary problems.

- Variables x and y are complementary to each other if both are non-negative but one is always zero in the pair. 
The complementarity notation is written compactly like this.

- Linear complementary problems involve finding x where x is complementary to some vector defined by Ax+b. Or simply to show that no such vector exists.

- A mixed linear complementary problem is a mixture of an LCP with a system of linear equations. Note that variable z here is not restricted to non-negativity.

Established root-finding approaches per Griewank et al., solve for the function f using a linear complementary problem.

Our group has developed a new root-finding approach that takes advantage of the triangularity of L to reformulate the complementarity problem with less restrictive assumptions. Unlike the established approach, this one doesn’t require the (i) same number of inputs as outputs, or for (ii) a nonsingular J. It also has fewer overall variables and equations.

Given the following newly defined matrices and vectors, the root of function f can be solved using the new indicated mixed linear complementarity problem. This is compatible with the PATH solver, which is an implementation of a stabilized Newton method for solving mixed-complementary problems.

However, if we can assume those restrictive assumptions again under this new problem setting and define these additional terms ρ and σ, then the problem actually simplifies to this linear complementarity problem!

Just as a sidenote, from our experience, J generally seems to be nonsingular when all the absolute variable terms z contribute in some meaningful way to f. That is, to produce non-zero answers. In fact, we’ve never actually seen a case yet where J is non-singular in some application. So, meeting that constraint is trivial.

Ok so that’s root-finding. Now, to minimize f instead, we can construct this equivalent linear program with complementarity constraints. This can be solved using a Branch-and-Cut approach.

Further, if we know the bounds on the optimal x and w, then we can use big-M constraints to rewrite this optimization program into a MILP.

Alrighty. Now to confirm the existence of an unconstrained global minima for any piecewise affine function, we need to examine horizon functions.

A horizon function is the mapping of some continuous function f that simply put describes how f behaves when the norm of x is large. It’s like zooming out infinitely far so that the function directional features are simplified.

So, for this example function, the solid line shows the original function where-as the horizon mapping shown by the dotted line flattens out its bumps and divots.

Ok, now note that the definition of the horizon function requires it to have a point at the origin. So, it if contains only positive direction vectors, like with this example here, its greatest lower bound is 0. However, if it contains even a single negative direction vector, the function goes off to negative infinity. Therefore, we can confirm the existence of a minima for any piecewise affine function by confirming if the horizon function contains any values below 0.

So, we set up an equivalent mixed-linear complementarity problem that tries to locate a solution for the horizon function equaling negative 1. For any piece-wise affine function, the horizon function is also piece-wise affine and can be represented in abs-normal form. So long as a solution to this problem doesn’t exist, we confirm no values exist below the 0 point, and that therefore, the original function has some global minima.

Our group has implemented these new complementarity formulations in Julia using PATH Solver and BARON.

- We can see how the root of this piecewise function can be located successively using both MLCP and LCP formulations.

- And how the both the existence of a global minimum and the minimum itself for this alternate function can also be located.

Ok now lets quickly go over the computational cost for root-solving problem of randomly generated piecewise affine functions.

- If J is a zero matrix, then PATH may fail to solve the complementarity problem at larger n whereas BARON does not. However, PATH outperforms BARON at all n.

- If J is nonsingular, then PATH may fail to solve when n is greater than 100. Do note that at this point, PATH can calculate the solution in less than the fraction of a second compared to BARON.

- The same can be said for the minimization of f for some variations of the piecewise function.

Ok! Our final section consists of root-finding formulations for ReLU neural networks. Here, we define a function g as the linear relationship between some variables x and y – where y is represented by a neural network.

Our goal here is to solve two main problem cases.

- The first involves setting P to 0 to simply solve for the root of the neural network.

This can be used to locate the steady state of a system or simply determine which inputs give certain outputs.

- The second will involve something called a neural ODE, which I’ll discuss shortly.

Alrighty, now the neural network architecture itself is a function composition of affine and activation functions. So, consider a network of L layers.

- We start with some initial nodes at layer 1.

- Each node on layer 2 is described as a linear combination of the nodes on the previous layers.

- Then each node is put through an activation function max0,x to add nonlinearity to the system.

Since the max function can be written using absolute notation, the neural network can be rewritten as a piecewise affine composite function. Therefore, we can represent the solution of function g using the indicated modified abs-normal form where we have redefined matrices J,Y, and vector b. This can be solved using the same complementarity approach introduced previously.

Now, our second problem case, neural ODEs. Neural ODEs are just an initial value ODE system whose derivatives are represented using neural networks. Neural networks are used when the problem dynamics are too complicated to represent directly mathematically.

- To solve this, we set P,Q,r accordingly to describe the implicit Euler method for a neural ODE. Solving g would then just be solving a root finding equation at N timesteps.

- Basically, consider some initial x value at time t0. The neural network is used to calculate the derivative at the next t1. This is then used to calculate x at that time t1. This process is repeated at each timestep.

The reason we use an implicit Euler approach is because explicit ODE solvers aren’t as numerically stable. Further, to our knowledge, the current state of implicit neural ODE solvers is underdeveloped.

We implemented these problem cases within Julia and this is just a quick summary of the construction within that implementation.

- We start off with some piece-wise affine representation of the neural network. We can partition those neural network parameters into the abs-normal form.

- If we have some residual function g, like the one required for neural ODE, we adapt the abs-normal form as shown.

- Then we take those adapted coefficients and construct the coefficients required for our new complementary solver and solve for the root x.

So, for our first problem case, our current implementation shows we can locate a local solution of the root for some piecewise affine functions.

And in our second problem case, given some set of user-defined neural network parameters representing some simple nonlinear ODE, it can automatically produce an ODE trajectory. This graph here shows the trajectory estimate is fairly accurate to the original raw data used to construct the neural ODE. Though keep in mind, this solver is only as good as the neural network parameters it has been provided.

In conclusion:

- Piece-wise affine approximations for nonsmooth functions are useful. They can be constructed using a forward-automatic differentiation procedure and represented in abs-normal form.

- We have presented new root finding and optimization approaches for these functions, with fewer variables and equations than the established approach.

- We have also presentation formulations for optimization problems with ReLU neural networks.

SPEECH TIME: 15 MINUTES